Year 10 – Student Booklet B

KS4 Forces 1

Lesson Breakdown

Lesson 1:  Classifying forces (4.5.1.1 & 4.5.1.2)

Lesson 2:  Gravity, mass & weight (4.5.1.3)

Lesson 3:  Resultant forces & resolving resulting forces (4.5.1.4)

Lesson 4:  Resolving resultant forces – Higher tier only

Lesson 5:  Work done (4.5.2)

Lesson 6:  Forces and elasticity (4.5.3) - theory

Lesson 7:  Required practical (4.5.3) - extension of a spring

Lesson 8: Required practical (4.5.3) - analysis

Lesson 9: Moments, levers and gears (4.5.4) - Physics only

Lesson 10: Pressure in fluids – Physics only

Lesson 11: Pressure at right angles to a surface – Physics only

Lesson 12: Atmospheric pressure – Physics only

Keystone words

- Vector

- Scalar

- Contact

- Non-contact

- Resultant

- Extension

Lesson 7: Investigating Hooke’s Law

Objective: You are learning about the relationship between force and the extension of a spring

Do It Now

| Answer |  | PA / SA |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |

I wasn’t there, but I still care.

Using the equation Work done (J) = Force (N) x distance (m)           (W = Fs), calculate:

- Calculate the work done if:

- F = 5 N, s = 5 m ____________________________________

- F = 150 N, s = 0.1 m	______________________________

- F = 0.2 N, s = 200 m  _______________________________

- F = 2000 N, s = 1.5 m	______________________________

- F = 800 N, s = 25 m	______________________________

Connect

- Which of the above images shows the force-extension graph for a soft spring? How can you tell?

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- What is meant by the term extension?

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Which equation links force (F), extension (e) and spring constant (k)?

_______________________________________________________________________________

- If a spring has a force of 10N applied to it which extends it by 60mm, what is its spring constant?

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Notes:

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Hooke’s Law:

Hooke’s Law is named English physicist Robert Hooke.  He performed a number of experiments on springs and elasticity in the 1600’s and found out that:

‘the extension of an elastic object (like a spring) is directly proportional to the force applied’

(directly proportional means that both variables increase at the same rate i.e. if one doubles, so does the other).

The full definition has a key bit of information on the end, so the full definition reads:

The extension of an elastic object is directly proportional to the force applied, provided that the limit of proportionality is not exceeded’.

Task 1:

Investigating Hooke’s Law.

Read the method given below carefully and identify the following:

- The independent variable: ________________________________________________________________________________________________________________________________________________________________

- The dependent variable:

________________________________________________________________________________________________________________________________________________________________

- Control variables:

________________________________________________________________________________________________________________________________________________________________

Method:

- Hang the spring on a clamp and add a splint so it’s easier to read the length.

- Making sure the splint is horizontal, take a reading on the ruler – this is the length of the unstretched spring (at Weight = 0.0 N)

- Carefully hook the 100g mass hanger onto the bottom of the spring.  This weighs 1.0 newton (1.0 N).

- Take a reading of the length of the spring.

- Add further masses.  Measure and record the length of the spring each time. Get at least 8 results.

- Calculate the extension i.e. the amount the string has stretched. To calculate this you subtract the length of the unstretched spring from each of your length readings.

The method given below is quite good because it includes all of the key steps in a logical order. However, it could be improved because it lacks some detail. This means that it would be hard for another person to reproduce the experiment without asking further questions.

On the lines under each step, include additional detail that would be useful for the experimenter.

Method:

- Hang the spring on a clamp and add a splint so it’s easier to read the length.

__________________________________________________________________________________________________________________________________________________________________

- Making sure the splint is horizontal, take a reading on the ruler – this is the length of the unstretched spring (at weight = 0.0 N).

__________________________________________________________________________________________________________________________________________________________________

- Carefully hook the 100g mass hanger onto the bottom of the spring.  This weighs 1.0 newton (1.0 N).

__________________________________________________________________________________________________________________________________________________________________

- Take a reading of the length of the spring.

__________________________________________________________________________________________________________________________________________________________________

- Add further masses.  Measure and record the length of the spring each time. Get at least 8 results.

__________________________________________________________________________________________________________________________________________________________________

- Calculate the extension i.e. the amount the string has stretched. To calculate this you subtract the length of the unstretched spring from each of your length readings.

__________________________________________________________________________________________________________________________________________________________________

| Hazard | Risk | Control/reduction measure |
| --- | --- | --- |

Use the diagram to the left to identify the main hazards in this experiment. Then complete the table below.

When assessing risk, consider the following criteria:

- 1. The way the hazard causes harm.

- 2. Likelihood of exposure to harm.

- 3. How serious the effects of the hazard would be.

Recording results: You will need to do each reading at least twice. This is so you can evaluate your data for reproducibility.

Use the following results table to record your results:

Trial 1:

| Mass (kg) | Force (N) | Length (mm) | Extension (mm) | Extension (m) |
| --- | --- | --- | --- | --- |

Trial 2:

| Mass (kg) | Force (N) | Length (mm) | Extension (mm) | Extension (m) |
| --- | --- | --- | --- | --- |

Trial 3:

| Mass (kg) | Force (N) | Length (mm) | Extension (mm) | Extension (m) |
| --- | --- | --- | --- | --- |

For each weight (force) added to the spring:

- Identify anomalies by circling them.

- Calculate the mean extension (remember to remove anomalies before calculating the mean).

- Obtain mean data from another group.

| Force (N) | Own data: Mean extension (m) | Data from another group: Mean extension (m) |
| --- | --- | --- |

Evaluating the quality of the data

- Repeatability: Look at the trials that you did for this experiment.

- Are the data repeatable? ______________________________________________________

- How do you know this? _______________________________________________________

__________________________________________________________________________

- Reproducibility: Compare your mean data to that from the other group.

- Are the data reproducible? ____________________________________________________

- How do you know this? _______________________________________________________

__________________________________________________________________________

- Precision: Compare the data for your trials to the mean value.

- Are the data precise? _________________________________________________________

- How do you know this? _______________________________________________________

__________________________________________________________________________

- Accuracy: Compare the data from your trials to the mean.

- Are the data accurate? ________________________________________________________

- How do you know this? _______________________________________________________

__________________________________________________________________________

- Anomalies: Look at your individual trials for each weight used.

- Did you have any anomalies? __________________________________________________

- If you had anomalies, what errors occurred during the experiments that could cause anomalies? _________________________________________________________________

___________________________________________________________________________

By referring to your answers to the questions above, decide whether your data are of high quality.

Are your data of high quality? ___________________________________________________________

Give at least three pieces of evidence that support your answer:

- _____________________________________________________________________________

- _____________________________________________________________________________

- _____________________________________________________________________________

- _____________________________________________________________________________

- _____________________________________________________________________________

Plot your data on the graph paper below.

- Force (weight) goes on the y-axis.

- Extension goes on the x-axis.

- You obtain the spring constant by calculating the gradient of the linear part of the graph.

Conclusion:

Describe the relationship between the independent and dependent variable.

__________________________________________________________________________________

Use data from the graph to support your conclusion.

____________________________________________________________________________________________________________________________________________________________________

Lesson 8: Analysing Required Practical

Objective: You are learning how to interpret Hooke’s Law data and use associated equations

Do It Now

| Answer |  | PA / SA |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |

I wasn’t there, but I still care.

Last lesson was a required practical.  Process the example results below:

Connect

Last lesson you completed the required practical to investigate Hooke’s law

- Define Hooke’s Law:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- What is the equation that links force (F), extension (e) and spring constant (k)?

_______________________________________________________________________________

- In what form is the energy stored in an extended or compressed spring?

______________________________________________________________________________________________________________________________________________________________

- What equation could be used to calculate the value of this energy store?

_______________________________________________________________________________

- What were the variables in your investigation:

Independent:___________________________________________________________________________________________________________________________________________________

Dependent:_____________________________________________________________________________________________________________________________________________________

Controls (x2):____________________________________________________________________

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Your investigation may have included sources of error.  Where would these errors have come from?

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Notes:

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Interpreting Hooke’s Law:

You graph may have looked like the following:

Hooke’s Law states that force and extension are directly proportional until the limit of proportionality is exceeded.  You may be able to identify the limit of proportionality in your data.  Look for when your spring constant value differs from your other values.  Once a spring passes its limit of proportionality it becomes elastically deformed.  This means that it will no longer return to its original shape and its original shape has been changed.

Applied forces change the shape of an elastic object.  This is either through extension or compression.  As such, when an elastic object, like a spring, is extended or compressed, work is done.  The stretched or compressed elastic object stores elastic potential energy.

Calculations using Hooke’s Law:

Hooke’s Law uses the following formula:

Force (N) = Spring constant (N/m) x extension (m)

And is abbreviated to:

F = ke

I do:

If the spring constant is 30 N/m and a spring is stretched by 0.3m, how much force has been applied?

- Write out the equation:   	F = k x e

- Fill in the data:		      	F = 30 x 0.3

- Complete calculation:	     	F = 9

- Add units:				F = 9N

We do:

If the spring constant is 12.6 N/m and a spring is stretched by 0.25m, how much force has been applied?

- Write out the equation:	F = k x e

- Fill in the data:			F = ________________________________

- Complete the calculation:	F = ________________________________

- Add units:				F = ________________________________

You do:

What force would be needed to extend a spring with a spring constant k = 10 N/m by an extension of 0.3 m?

- Write out the equation: ______________________________________

- Fill in the data: _____________________________________________

- Complete the calculation: ____________________________________

- Add units: _________________________________________________

The calculation can be rearranged to make either spring constant or extension the subject:

Spring constant =___________________________________________________

Extension =________________________________________________________

Complete:

Calculate the spring constant if:

- F = 150 N, e = 0.075 m.

- Write out the equation: ____________________________________

- Fill in the data: ___________________________________________

- Complete the calculation: __________________________________

- Add units: _______________________________________________

- F = 50 N, e = 0.1 m.

- Write out the equation: ____________________________________

- Fill in the data: ___________________________________________

- Complete the calculation: __________________________________

- Add units: _______________________________________________

Calculate the extension if:

- F = 15 N, k = 150 N/m.

- Write out the equation: ____________________________________

- Fill in the data: ___________________________________________

- Complete the calculation: __________________________________

- Add units: _______________________________________________

- F = 45 N, k = 90 N/m.

- Write out the equation: ____________________________________

- Fill in the data: ___________________________________________

- Complete the calculation: __________________________________

- Add units: _______________________________________________

The energy stored by elastic objects can also be calculated.  You first met this equation in the energy unit:

Elastic energy (J) = ½ x spring constant (N/m) x extension2 (m)

Ee = ½ke2

I do:

Spring constant k = 5 N/m, spring extension e = 0.1 m.

- Write out the equation:	Ee = ½ x k x e2

- Fill in the data:			Ee = 0.5 x 5 x 0.12

- Square e:				Ee = 0.5 x 5 x 0.01

- Complete calculation:		Ee =0.025

- Add units:				Ee = 0.025J

You do:

Spring constant k = 20 N/m, spring extension e = 0.15 m.

- Write out the equation:	Ee = ½ x k x e2

- Fill in the data:			Ee =_______________________________

- Square e:				Ee =_______________________________

- Complete calculation:		Ee =_______________________________

- Add units:				Ee = _______________________________

Complete:

A spring with k = 450 N/m is compressed by 0.13m. How much energy is stored?

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Multistep equations (HT)

There is the expectation that you can complete a multistep equation using both F=ke and Ee = ½ke2 to calculate the energy contained in a stretched or compressed elastic object using experimental data.

An example:

A spring is extended by 0.2m when a force of 20N is applied.  How much energy is stored by the spring?

To determine the energy value, we need to use Ee = ½ke2, however the question does not include a value for k (spring constant).  This means we first have to calculate spring constant using F = ke, and then calculate energy stored using Ee = ½ke2:

- Write out equation: 	k = F ÷ e

- Fill in the data:		k = 20 ÷ 0.2

- Complete calculation: k = 100N/m

Now we can calculate elastic energy stored:

- Write out equation:	Ee = ½ x k x e2

- Fill in the data:		Ee = 0.5 x 100 x 0.22

- Square e:			Ee = 0.5 x 100 x 0.04

- Complete calculation:	Ee = 2J

Multistep equations SLOP

A spring has a weight of 50N hanging on it, and is stretched from a length of 0.05m to a length of 0.15 m. How much energy is stored by the spring?

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

A spring has a weight of 20N hanging on it, and is stretched from a length of 0.20m to a length of 0.45m. How much energy is stored by the spring?

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Q1.

A student investigated how the extension of a spring varied with the force acting on the spring.

The diagram below shows the equipment the student used and a ruler scale between 10 cm and 15 cm

(a)     Describe how the student should determine the extension of the spring.

_______ ____________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(2)

(b)     Write down the equation which links extension (e), force (F) and spring constant (k).

______________ _____________________________________________________

(1)

(c)     The extension of the spring was 0.12 m when the force was 3.0 N

Calculate the spring constant of the spring.

___________________________________________________________________

_____________ ______________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Spring constant = _______________ N/m

(3)

(d)     Determine the energy stored by the spring using your calculation from (c)

Use the Physics Equations Sheet.

______________ _____________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Energy stored = _______________ J

(3)

(Total 9 marks)

Extra exam practice

A student investigated the behaviour of springs. She had a box of identical springs.

(a)     When a force acts on a spring, the shape of the spring changes.

The student suspended a spring from a rod by one of its loops. A force was applied to the spring by suspending a mass from it.

Figure 1 shows a spring before and after a mass had been suspended from it.

Figure 1

| Before |  | After |
| --- | --- | --- |

(i)      State two ways in which the shape of the spring has changed.

1. ______ ______________________________________________________

2. ____________________________________________________________

(2)

(ii)     No other masses were provided.

Explain how the student could test if the spring was behaving elastically.

______________________________________________________________

_______ _______________________________________________________

______________________________________________________________

______________________________________________________________

(2)

(b)     In a second investigation, a student took a set of measurements of force and extension.

Her results are shown in Table 1 .

Table 1

| Force in newtons | 0.0 | 1.0 | 2.0 | 3.0 | 4.0 | 5.0 | 6.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Extension in cm | 0.0 | 4.0 |  | 12.0 | 16.0 | 22.0 | 31.0 |

(i)      Add the missing value to Table 1.

Explain why you chose this value.

_________ _____________________________________________________

______________________________________________________________

______________________________________________________________

______________________________________________________________

(3)

(ii)     During this investigation the spring exceeded its limit of proportionality.

Suggest a value of force at which this happened.

Give a reason for your answer.

Force = _________________ N

Reason _______ ________________________________________________

______________________________________________________________

______________________________________________________________

(2)

(c)     In a third investigation the student:

•        suspended a 100 g mass from a spring

•        pulled the mass down as shown in Figure 2

•        released the mass so that it oscillated up and down

•        measured the time for 10 complete oscillations of the mass

•        repeated for masses of 200 g, 300 g and 400 g.

Figure 2

Her results are shown in Table 2.

Table 2

|  | Time for 10 complete oscillations in seconds |  |  |  |
| --- | --- | --- | --- | --- |
| Mass in g | Test 1 | Test 2 | Test 3 | Mean |
| 100 | 4.34 | 5.20 | 4.32 | 4.6 |
| 200 | 5.93 | 5.99 | 5.86 | 5.9 |
| 300 | 7.01 | 7.12 | 7.08 | 7.1 |
| 400 | 8.23 | 8.22 | 8.25 | 8.2 |

(i)      Before the mass is released, the spring stores energy.

What type of energy does the spring store?

Tick () one box.

|  | Tick () |
| --- | --- |
| Elastic potential energy |  |
| Gravitational potential energy |  |
| Kinetic energy |  |

(1)

(ii)     The value of time for the 100 g mass in Test 2 is anomalous.

Suggest two likely causes of this anomalous result.

Tick () two boxes.

|  | Tick () |
| --- | --- |
| Misread stopwatch |  |
| Pulled the mass down too far |  |
| Timed half oscillations, not complete oscillations |  |
| Timed too few complete oscillations |  |
| Timed too many complete oscillations |  |

(2)

(iii)    Calculate the correct mean value of time for the 100 g mass in Table 2.

_______ _______________________________________________________

______________________________________________________________

Mean value = __________________ s

(1)

(iv)    Although the raw data in Table 2 is given to 3 significant figures, the mean values are correctly given to 2 significant figures.

Suggest why.

__________ ____________________________________________________

______________________________________________________________

______________________________________________________________

(2)

(v)     The student wanted to plot her results on a graph. She thought that four sets of results were not enough.

What extra equipment would she need to get more results?

_________ _____________________________________________________

______________________________________________________________

______________________________________________________________

______________________________________________________________

(2)

(Total 17 marks)

Lesson 9: Moments, levers and gears (4.5.4) – Physics only

Objective: Today we are learning about moments (rotational effects of forces), and how they relate to gears and levers.

Do It Now

| Answer |  | PA / SA |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |

I wasn’t there, but I still care.

The last lesson involved the analysis of results from the Hooke’s law required practical. Use the AQA Physics textbook (page 158-159) to answer the following questions:

- What happens to the extension of a spring as more force is added?

______________________________________________________________________________________________________________________

- What is the relationship between force and extension?

___________________________________________________________

- What eventually happens to a spring if too much force is added? Why does this happen?

_________________________________________________________________________________________________________________________________________________________________________________

Connect

You have looked at moments previously in the KS3 Work topic. You have seen that a moment is: “the turning effect of a force around a fixed point or axis”.

- The size of a moment depends on two factors. What are these two factors? State the units they are measured in.

______________________________________________________________________________________________

- Recall the equation that connects moments with the two factors stated above.

_______________________________________________

- In terms of forces, how does a lever make work easier?

_____________________________________________________________________________________________________________________________________________

Notes:

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Modelling progress

A moment is the turning effect of a force around a fixed point or axis. It is defined by the equation below:

Moment Nm=size of the force N x perpendicular distance of force from pivot (m)

This means that increasing either the size of the force or the perpendicular distance of that force from the pivot will increase the moment.

I DO

Calculate the moment when a force of 300 N is applied to a seesaw, 0.5 m from the pivot.

STEP 1: Write out the equation: ________________________________________________

STEP 2: Substitute in the values: _______________________________________________

STEP 3: Calculate the answer with units: _________________________________________

WE DO

Calculate the moment on a door when it is closed with a force of 25 N. The door handle is 1.5 m away from the hinges.

STEP 1: Write out the equation: ________________________________________________

STEP 2: Substitute in the values: _______________________________________________

STEP 3: Calculate the answer with units: _________________________________________

YOU DO (task 1)

- Calculate the moment in each of the following cases:

- _________________________________________________________________________________________________________________________________________________________________________________

- _________________________________________________________________________________________________________________________________________________________________________________

- _________________________________________________________________________________________________________________________________________________________________________________

- _________________________________________________________________________________________________________________________________________________________________________________

- Rearrange the equation to calculate:

  - The size of the force when a moment of 250 Nm is produced 12.5 m from the pivot.

__________________________________________________________________________________________________________________________________________________________________________________________

  - The perpendicular distance from the pivot when a moment of 76 Nm is produced from a 12 N force.

__________________________________________________________________________________________________________________________________________________________________________________________

- Multiple moments can act on the same object, in different directions.

  - Calculate both the clockwise and counter-clockwise moments on the see-saw below:

Clockwise moment: __________________________________________ ___________________________________________________________

Counter-clockwise moment: ____________________________________ ___________________________________________________________

  - Would this see-saw balance? Explain your answer.

_____________________________________________________________________________________________________________________________________________________________________

Clockwise moment

Counter-clockwise moment

Modelling progress

Levers and gears transmit the rotational effects of forces (moments). This means they reduce the amount of work we need to do when turning objects. By reducing the amount of work needed to turn objects, less force is needed to turn objects.

Gears

Examples of levers:

YOU DO (task 2) – Complete these exam questions relating to levers:

Q1. The diagram shows a crane which is loading containers onto a ship.

(a)     Calculate the moment of the container which is being loaded.

Show clearly how you work out your answer and give the unit.

___________________________________________________________________

___________________________________________________________________

Moment of the container = ____________________________

(3)

(b)     Suggest and explain the purpose of the large concrete blocks.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(3)

(Total 6 marks)

Q2.

(a)     Figure 1 shows a girl standing on a diving board.

Figure 1

Calculate the total clockwise moment of the weight of the diving board and the weight of the girl about Point A. Give the unit.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Total clockwise moment about Point A = __________________________________

(4)

(b)     Figure 2 shows the girl standing at a different place on the diving board.

The support provides an upward force F to keep the diving board balanced.

Figure 2

Figure 3 shows how the upward force F varies with the distance of the girl from Point A.

Figure 3

Distance of girl from Point A in metres

Explain, in terms of clockwise and anticlockwise moments, why the upward force F increases as shown in Figure 3.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(3)

(Total 7 marks)

Lesson 10: Pressure in Fluids

Objective: I can explain what happens to the pressure in a liquid the deeper you go into the liquid and calculate pressure at different depths.

Do It Now

| Answer |  | PA / SA |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |

I wasn’t there, but I still care.

A force or system of forces may cause an object to turn. A moment is the turning effect of a force. Moments act about a point in a clockwise or anticlockwise direction. ​

The point chosen could be any point on the object, but the pivot - also known as the fulcrum - is usually chosen.

The magnitude (size) of a moment can be calculated using the equation:​

​

moment of a force = force × distance​

​

M = Fxd​

​

This is when:​

moment (M) is measured in newton-metres (Nm)​

force (F) is measured in newtons (N)​

distance (d) is measured in metres (m)​

- A force of 15 N is applied to a door handle, 12 cm from the pivot. Calculate the moment of the force.

_________________________________________________________________________________________________________________________________________________________________________________________________________

- A force of 40 N is applied to a spanner to turn a nut. The perpendicular distance is 30 cm. Calculate the moment of the force.

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Connect

Fluids are substances that flow.  Liquids and gases are both fluids.​

​

When there is a pressure in a fluid a force is produced at right angles to the surface containing the fluid.​

The size of the force acting at the surface of a fluid can be calculated using the equation:​

Pressure (Pa) = Force normal to a surface (N)​

Area of that surface (m²)​

p = 𝑭/𝑨​

I Do

Work out the pressure in a fluid when the fluid applies a force of 12 N on a surface of area 0.02 m².  __________________________________________________________________________________________________________________________________________________

We Do

In a bath full of water a force of 1250 N acts on an area of 0.5m²at the bottom of the bath.  Calculate the pressure acting on the bottom of the bath.

__________________________________________________________________________________________________________________________________________________

You Do

A force of 18 N acts on a surface that has an area of 0.015 m².  Work out the pressure acting on the surface.

__________________________________________________________________________________________________________________________________________________

Notes:

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Higher

The pressure in a liquid is different at different depths. Pressure increases as the depth increases. The pressure in a liquid is due to the weight of the column of water above. Since the particles in a liquid are tightly packed, this pressure acts in all directions. For example, the pressure acting on a dam at the bottom of a reservoir is greater than the pressure acting near the top. This is why dam walls are usually wedge-shaped.

Watch this Demonstration.

Explain why the water at the bottom of the bottle travels further than the water at the top of the bottle?​

In your answer use the words:​

Speed​

Pressure​

particles​

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

As you go down a column of water the pressure at the bottom of the column is greater than the pressure at the top of the column.  There is more weight of water above you if you are at the bottom of the column.  ​

​

The pressure at a certain depth in a liquid depends on the height of liquid above the base (h), the density of a liquid (⍴) and the gravitational field strength (g)​

The pressure caused by a column of liquid can be calculated using the equation: ​

​

pressure = height of column × density of the liquid × gravitational field strength​

p = h⍴g​

This is when:  pressure (p) is measured in pascals (Pa)​

height of column (h) is measured in metres (m)​

density (ρ) is measured in kilograms per metre cubed (kg/m3)                                    ​

gravitational field strength (g) is measured in newtons per kilogram (N/kg)​

The top of a submarine is 50 m underwater.  The density of seawater is 1030 kg/m³.

Work out the pressure on the top of the submarine.  Take g = 9.8 N/kg.

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

A floating object experiences a greater pressure on the bottom surface than on the top of the surface.  This creates a resultant force upwards.  This force is called upthrust.  ​

When solid objects are placed in water they will float or sink depending on the density of the solid material compared to the density of the water.  ​

​

If the object is denser than water, it will sink.  This is because the weight is greater than upthrust.​

If the object is less dense than water, it will float because the upthrust is greater than weight.​

​

Submarines can adjust the depth they float at by adding or removing water from ballast tanks.  Scuba divers use air in a jacket to control their depth.  Fish have swim bladders which serve a similar purpose.

- A beaker is filled to a depth of 10 cm with water.  Water has a density of 1000 kg/m3.  Calculate the pressure acting at the bottom of the beaker.

Take g = 9.8 N/kg.​

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- A scuba diver is diving in the sea.  The pressure acting on the scuba diver is 267 800 Pa. Salt water has a density of 1030 kg/m3.  Calculate the depth of the scuba diver.​

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Imagine taking a sealed tin can on a dive.  As you take it down deeper what​

would happen to the pressure, and the can, if you take it deep enough?​

​

​____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

4.  Explain how a submarine can move up and down in the water.  Use the​ following keywords:  density, weight, pressure, particles, upthrust.

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

5. Calculate the upthrust on a submarine that has a height of 15 m and is submerged at a depth of 60 m (to the top of the submarine).  The submarine has a surface area of 100 m² on the top and on the bottom of the submarine.  ​

​

Take the density of sea water to be 1100 kg/m³ and the gravitational field strength of the Earth to be 9.8 N/kg.

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Lesson 11: Pressure at right angles to a surface – Physics only

Objective: Today we will be learning that pressure in fluids causes a force normal to any surface, and how to calculate this pressure.

Do It Now

| Answer |  | PA / SA |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |

I wasn’t there, but I still care.

Last lesson we learned about pressure in fluids, how to calculate it, and the effect of column height and density on pressure. Use the AQA Physics text book (page164-165) to answer the following questions.

- What happens to the pressure in a liquid as the liquid depth increases?

____________________________________________________

- Describe the pressure along a horizontal line in a liquid.

________________________________________________________________________________________________________

- Describe the relationship between density and pressure in a liquid.

________________________________________________________________________________________________________

Connect

We have looked at pressure in fluids previously in this topic.

You will have learned how to calculate area in maths.

Answer the following questions:

- What equation is used to calculate the pressure due to a column of liquid?

___________________________________________________________

- How would you calculate the surface area of a square/rectangle?

___________________________________________________________

- What is a “right-angle”?

___________________________________________________________

- Calculate the area of the surfaces below:

- __________________________________

__________________________________

__________________________________

- __________________________________

__________________________________

__________________________________

Notes:

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Modelling progress

In science, fluids can be considered to be either liquids or gases. When a fluid causes pressure on a surface, this pressure is normal (at a right angle/90o) to that surface. We can calculate the pressure on a surface by using the following equation:

Pressure on a surface (Pa)=Force (N)  x surface area (m2)

Pressure on a surface à Pascals (Pa)

Force à Newtons (N)

Surface area à Metres squared (m2)

I DO

Calculate the pressure on a surface when 500 N of force is applied against a surface with an area of 12.5 m2.

STEP 1: Write down the equation: _____________________________________

STEP 2: Substitute in the values: ______________________________________

STEP 3: Calculate the answer with units: ________________________________

WE DO

Calculate the pressure on a surface when 160 N of force is applied against a surface with an area of 2.6 m2.

STEP 1: Write down the equation: _____________________________________

STEP 2: Substitute in the values: ______________________________________

STEP 3: Calculate the answer with units: ________________________________

YOU DO (task 1)

Calculate the pressure on the following surfaces. Calculate the surface area if needed using the equation: surface area = length x width.

- A force of 230 N is applied to this surface:

___________________________________

___________________________________

___________________________________

___________________________________

- A force of 112 N is applied to this surface:

____________________________________

____________________________________

____________________________________

- A force of 75 N is applied to this surface:

___________________________________

___________________________________

___________________________________

___________________________________

- A force of 96.7 N is applied to this surface:

___________________________________

___________________________________

___________________________________

___________________________________

Rearrange the equation to calculate the size of the force in the following questions:

- A pressure of 680 Pa against a surface with an area of 6.5 m2.

_________________________________________________________________________________________________________________________________________________________________________________

- A pressure of 45.6 Pa against a surface with an area of 1.5 m2.

_________________________________________________________________________________________________________________________________________________________________________________

- A pressure of 6.5 kPa against a surface with an area of 30 m2.

_________________________________________________________________________________________________________________________________________________________________________________

- A pressure of 0.9 kPa against a surface with an area of 2.2 m2.

_________________________________________________________________________________________________________________________________________________________________________________

YOU DO (task 2) – Complete these exam questions relating to pressure at right angles:

Q1. Mountain bike riders use brakes to slow down.

© Ljupco Smokovski/Shutterstock

Some mountain bikes use liquid-filled pipes to transmit the force from the rider’s hand on the brake lever to the brake pads. These brakes are called hydraulic brakes.

(a)     Draw a ring around the correct answer to complete each sentence.

(i)      Liquids can be used to transmit the forces in a brake system,

|  | are incompressible. |
| --- | --- |
| because liquids | can flow. |
|  | take the shape of the container. |

(1)

(ii)

|  | against force F only. |
| --- | --- |
| The pressure in the liquid is transmitted | downwards only. |
|  | in all directions. |

(1)

(b)     When the rider’s hand pulls on the brake lever, the force F applied to the liquid by the master piston is 80 N. The cross-sectional area of this piston is 50 mm2.

Calculate the pressure, in N/mm2, exerted on the liquid by the master piston.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Pressure = _______________ N/mm2

(2)

(c)     The unit N/mm2 is not the usual unit of pressure.

Which unit is usually used when calculating pressure?

Draw a ring around the correct answer.

| N | Nm2 | Pa |
| --- | --- | --- |

(1)

(d)     The rider applies a larger force to the brake lever. How would this increase in force affect the pressure in the liquid?

___________________________________________________________________

(1)

(Total 6 marks)

Q2. Figure 1 shows how atmospheric pressure varies with altitude.

Figure 1

(a)     Explain why atmospheric pressure decreases with increasing altitude.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(3)

(b)     When flying, the pressure inside the cabin of an aircraft is kept at 70 kPa. The aircraft window has an area of 810 cm2.

Use data from Figure 1 to calculate the resultant force acting on an aircraft window when the aircraft is flying at an altitude of 12 km.

Give your answer to two significant figures

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Resultant force = ___________________ N

(5)

(c)     Figure 2 shows the cross-section of one type of aircraft window.

Figure 2

Explain why the window has been designed to have this shape.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(2)

(Total 10 marks)

Lesson 12: Atmospheric Pressure

Objective: I can describe a simple model of the Earth’s atmosphere and of atmospheric pressure, and I can explain why atmospheric pressure varies with height above a surface.

Do It Now

| Answer |  | PA / SA |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |

I wasn’t there, but I still care.

The diagram shows a container filled with water.  The three holes in the side of the container are sealed with rubber stoppers.  The water exerts a force of 27 N on the bottom of the container.  The cross-sectional area of the bottom of the container is 0.009 m2.

Calculate the pressure exerted by the water on the bottom of the container.

Use the equation:

Choose the unit.

| kg/m3 | N/m | Pa |
| --- | --- | --- |

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Pressure = ______________________ Unit = __________

(3)

Connect

The Earth’s atmosphere can be thought of as a column of particles many kilometres high above a point on the ground.  These particles are pulled towards the centre of the Earth by its gravitational field.  This weight of particles is known as the atmospheric pressure.  A person standing on the ground is being has a pressure of 1 atmosphere on them.  The higher a person or object travels above the Earth’s surface (for example to the top of a high mountain or in an aeroplane) the fewer particles there are above them.  This means that the pressure is reduced.  When the pressure is less, the particles move further apart.  In everyday language we say the “air is thinner”, we mean that there are fewer particle in the same volume.  This means that a mountaineer would get less oxygen in each breath when they are at altitude.

Notes:

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Demonstration

Watch the demo of a collapsing can and explain why the can collapses once turned over.  In your answer use the keywords:   particles, pressure, density​.

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

You Do Questions

Q1.

The Earth is surrounded by an atmosphere.

(a)  The radius of the Earth is 6400 km.

Which of the following could be an approximate depth of the Earth’s atmosphere?

Tick (✓) one box.

| 100 km |  |
| --- | --- |
| 6400 km |  |
| 100 000 km |  |
| 640 000 km |  |

(1)

(b)  What state of matter is most of the Earth’s atmosphere?

Tick (✓) one box.

| Gas |  |
| --- | --- |
| Liquid |  |
| Solid |  |

(1)

Figure 1 shows how atmospheric pressure varies with height above sea level.

Figure 1

(c)  The highest point above sea level in England is the top of a mountain called Scafell Pike.

The height above sea level of Scafell Pike is 978 m.

Determine the atmospheric pressure at the top of Scafell Pike.

Use Figure 1.

___________________________________________________________________

___________________________________________________________________

Atmospheric pressure = _______________ Pa

(1)

(d)  Determine the difference between the atmospheric pressure at sea level and at the top of Scafell Pike.

Use Figure 1 and your answer from part (c)

___________________________________________________________________

___________________________________________________________________

Difference in atmospheric pressure = _______________ Pa

(1)

(e)  A student climbs Scafell Pike.

Why does the atmospheric pressure decrease as the student climbs higher?

Tick (✓) two boxes.

| The air exerts a greater force on the student. |  |
| --- | --- |
| The density of the air decreases. |  |
| The mass of air above the student decreases. |  |
| The temperature of the air increases. |  |
| The volume of air above the student increases. |  |

(2)

(f)  Figure 2 shows a mountain lake.

Figure 2

The lake has a surface area of 2000 m2.

Atmospheric pressure exerts a force of 188 000 000 N on the surface of the lake.

Calculate the atmospheric pressure at the surface of the lake.

Use the equation:

pressure =

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Atmospheric pressure = _______________ Pa

(2)

(Total 8 marks)

Q2.

An aeroplane is 4000 m above the Earth’s surface.

A skydiver jumps from the aeroplane and falls vertically.

Figure 1 shows the distance the skydiver falls during the first 12 seconds after jumping.

Figure 1

(a)  Figure 2 shows part of the free body diagram for the skydiver three seconds after jumping.

Complete the free body diagram for the skydiver.

Figure 2

(2)

(b)  Explain the changing motion of the skydiver in terms of the forces acting on the skydiver.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(4)

(c)  Use Figure 1 to determine the speed of the skydiver between 7 seconds and 12 seconds.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Speed = __________________________ m/s

(d)  In 2012 a skydiver jumped from a helium balloon 39 000 metres above the Earth’s surface. The skydiver reached a maximum speed of 377 m/s

Jumping from 39 000 metres allowed the skydiver to reach a much higher speed than a skydiver jumping from 4000 metres.

Explain why.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(3)

(Total 12 marks)

Q3.

Figure 1 shows how atmospheric pressure varies with altitude.

Figure 1

(a)     Explain why atmospheric pressure decreases with increasing altitude.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(3)

(b)     When flying, the pressure inside the cabin of an aircraft is kept at 70 kPa.

The aircraft window has an area of 810 cm2.

Use data from Figure 1 to calculate the resultant force acting on an aircraft window when the aircraft is flying at an altitude of 12 km.

Give your answer to two significant figures

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Resultant force = ___________________ N

(5)

(c)     Figure 2 shows the cross-section of one type of aircraft window.

Figure 2

Explain why the window has been designed to have this shape.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(2)